# FreeScan Series – SDK User Manual

Version 1.0.0

# Revision Record

| Version | Modification | Date |
| --- | --- | --- |
| 0.0.1 | First draft generation | December 28, 2023 |
| 0.0.2 | Added chapter on calling processes | January 4, 2024 |
| 1.0.0 | New formatted and translated to English | February 22, 2024 |

# Contents

Revision Record	2

Contents	3

1	SDK Invocation Process	5

2	Data Structure and Macro Definitions	7

Data Structure	7

Scene Data	7

Marker	7

Point Cloud	7

Point Data	8

Scanner RT	8

Mesh Data	8

Image Data	9

Triangulated Data	9

UV Coordinates	9

Increase Mesh Data	9

Scan Parameters	10

FreeScan Mesh Parameters	10

FreeScan Save Parameters	10

Macro Definitions	11

Device Type	11

Error Code	11

Scanning Mode	12

Scanning Status	13

Object Type	13

3	Interface Definitions for Handheld Scanner	14

Initializing a Device Handle	14

Releasing a Device Handle	14

Registering Callbacks	15

Callback response function	15

Messages supported by FreeScan devices	16

Send command	18

Messages supported by FreeScan devices	19

Set calibration mode	23

Start calibration	24

End calibration	24

Create new project	24

Set scanning parameters	25

Set laser line level	25

Set whether to add frame points	26

Start scanning	26

Pause scanning	26

Cancel scanning	27

End scanning	27

Mesh	27

Confirm mesh	28

Save mesh	28

Set scan mode	29

Enter scan page	29

Enter scan mode page	29

Set brightness	30

Select high-speed scanning mode	30

Set scan object	30

Register scene data callback	31

Get scene data	31

Video data analysis	33

Start process video data	34

Stop process video data	34

# SDK Invocation Process

Start by completing the environment configuration as outlined in the document titled "XL-QER-RD-012 SDK 2.0 Development Environment Configuration SDK 2.0.docx."

Next, invoke the interface functions defined in “SDKCWrapper.h”.

Process the subsequent application logic based on the return value and the result callback function. Refer to the definition of the handheld scanning interface for guidance.

Figure 1: Call process diagram

|  | NOTE Sending command and callback are two independent logical processes that do not interfere with each other. This process resembles common network programs, where the sending module is responsible solely for sending and informing about the success or failure of the transmission. The receiving (callback) process is responsible for retrieving the current system result, which may include device availability, execution results of specific instructions, available system memory, etc. |
| --- | --- |

Figure 2 : Call process diagram

|  | NOTICE Sending command and callback are two independent logical processes that do not interfere with each other. This process resembles common network programs, where the sending module is responsible solely for sending and informing about the success or failure of the transmission. The receiving (callback) process is responsible for retrieving the current system result, which may include device availability, execution results of specific instructions, available system memory, etc. |
| --- | --- |

# Data Structure and Macro Definitions

## Data Structure

### Scene Data

SN3D_SCENE_DATA

| Member Variables | Meaning |
| --- | --- |
| nCurrentMarkersCount | Current number of flag points |
| nWholeMarkersCount | Total number of flag points |
| nCurrentCloudCount | Current number of cloud points |
| nWholeCloudCount | Total number of cloud points |
| pCurrentMarkers | Pointer to current flag points array, type SN3D_MARKER |
| pWholeMarkers | Pointer to overall flag points array, type SN3D_MARKER |
| pCurrentCloud | Pointer to current point cloud array, type SN3D_CLOUDPOINT |
| pWholeCloud | Pointer to overall point cloud array, type SN3D_CLOUDPOINT |
| aCurrentCloudRt | Rotation matrix and translation vector, type RT structure |
| mMesh | Overall mesh, type SN3D_MESH_DATA |
| mIncreaseMesh | Incremental mesh, type SN3D_INCREASE_MESH_DATA |

### Marker

SN3D_MARKER

| Member Variable | Meaning |
| --- | --- |
| mCoords | Array of flag point normal vectors (length of 3 float types) |
| mNormals | Array of flag point normal vectors (length of 3 float types) |
| nId | ID number |
| nDiameter | Diameter (float type) |
| nCodeId | Encoding point ID (integer type), where ID less than 0 indicates a regular flag point |
| bIsDelete | Boolean indicating whether to delete or modify flag points |

### Point Cloud

SN3D_CLOUDPOINT

| Member Variable | Meaning |
| --- | --- |
| pPointCoords | Array of point cloud coordinates, type SN3D_POINT_DATA |
| pPointNormals | Array of point cloud normal vectors, type SN3D_POINT_DATA |
| pPointColors | Array of point cloud texture colours, type SN3D_POINT_DATA |

### Point Data

SN3D_POINT_DATA

| Member Variable | Meaning |
| --- | --- |
| nX | X-coordinate (float type) |
| nY | Y-coordinate (float type) |
| nZ | Z-coordinate (float type) |

### Scanner RT

SN3D_SCANNER_RT

| Member Variable | Meaning |
| --- | --- |
| aRot | Array of rotation matrices (9 float types) |
| aTrans | Array of translation vectors (3 float types) |

### Mesh Data

SN3D_MESH_DATA

| Member Variable | Meaning |
| --- | --- |
| nMeshpointCount | Length of vertex array |
| nMeshNormalCount | Length of normal vector array |
| nMeshTriFaceIdCount | Length of point cloud index array |
| nMeshTextureIdCount | Length of texture index array |
| nTextureUVCount | Length of texture UV coordinate array |
| pMeshPoint | Array of vertices, each element is of type SN3D_POINT_DATA representing (x,y,z) coordinates |
| pMeshNormal | Array of normal vectors, each element is of type SN3D_POINT_DATA representing vectors perpendicular to faces |
| pMeshTriFaceId | Array of point cloud index, each element is of type SN3D_TRI_FACE representing indices of 3 vertices |
| pMeshTextureId | Array of texture index, each element is of type SN3D_TRI_FACE |
| pTextureUV | Array of texture UV coordinates, each element is of type SN3D_UVCOORD |
| mImage | Texture image, type SN3D_IMAGE_DATA |

### Image Data

SN3D_IMAGE_DATA

| Member Variable | Meaning |
| --- | --- |
| width | Width of the texture image |
| height | Height of the texture image |
| channel | Number of channels in the texture image |
| data | Pointer to the data of the texture image (unsigned char*) |

### Triangulated Data

SN3D_TRI_FACE

| Member Variable | Meaning |
| --- | --- |
| x | Index of vertex x |
| y | Index of vertex y |
| z | Index of vertex z |

### UV Coordinates

SN3D_UVCOORD

| Member Variable | Meaning |
| --- | --- |
| uu | u-coordinate |
| vv | v-coordinate |

### Increase Mesh Data

SN3D_INCREASE_MESH_DATA

| Member Variable | Meaning |
| --- | --- |
| nVertexCount | Number of vertices |
| pVertex | Array of vertices, each element is of type SN3D_POINT_DATA |
| nMeshTriFaceIdCount | Temporary placeholder, memory allocation required |
| pMeshTriFaceId | SN3D_TRI_FACE type, temporary placeholder, memory allocation required |
| pWeightSet | Array of weights for each vertex, used for quality color spectrum |

### Scan Parameters

ScanPars

| Member Variable | Meaning |
| --- | --- |
| scanPointCloud | Boolean indicating whether to scan point cloud or mesh |
| scanMarkers | Boolean indicating whether to scan frame points (markers) |
| scanPhotographic | Boolean indicating whether to perform photogrammetry |

### FreeScan Mesh Parameters

FreeScanMeshPar

| Member Variable | Meaning |
| --- | --- |
| filter_level | Filtering level (integer type) |
| remove_small_floating_parts | Size of small floating parts to be removed, reference to client software |
| max_trianlges_ischeck | Boolean indicating whether to limit the number of triangles |
| max_trianlges_value | Maximum number of triangles (integer type) |
| fill_small_hole_ischeck | Boolean indicating whether to detect and fill small holes |
| fill_small_hole_value | Size of small holes to be filled, reference to client software |
| remove_spike_ischeck | Boolean indicating whether to remove small spikes |
| marker_hole_filling_ischeck | Boolean indicating whether to detect small holes formed by markers |
| isRememberChoice | Boolean indicating temporary non-usage |

### FreeScan Save Parameters

FreeSaveMeshPar

| Member Variable | Meaning |
| --- | --- |
| fileName | File name (const char* type) |
| folderPath | Folder name where the file is located (const char* type) |
| format | Format for validation; generate a string like "01000000" based on saveAscFile, etc. (const char* type) |
| method | Input "save" (const char* type) |
| postpageVisible | Internal use; set to true (bool type) |
| saveAscFile | 1 to save in asc format, 0 to not save (int type) |
| saveStlFile | 1 to save in stl format, 0 to not save (int type) |
| saveObjFile | 1 to save in obj format, 0 to not save (int type) |
| savePlyFile | 1 to save in ply format, 0 to not save (int type) |
| saveP3File | 1 to save in p3 format, 0 to not save (int type) |
| save3MfFile | 1 to save in 3mf format, 0 to not save (int type) |
| saveTxtFile | 1 to save in txt format, 0 to not save (int type) |
| saveCsvFile | 1 to save in csv format, 0 to not save (int type) |

## Macro Definitions

### Device Type

| Member Variable | Macrodefined value | Meaning |
| --- | --- | --- |
| SNSDK_DEVTYPE_UE | 0x00000000 | UE |
| SNSDK_DEVTYPE_UEPRO | 0x00000001 | UE Pro |
| SNSDK_DEVTYPE_COMBO | 0x00000002 | Combo |
| SNSDK_DEVTYPE_TRAK | 0x00000003 | Trak |
| SNSDK_DEVTYPE_5M | 0x00000004 | 5M |
| SNSDK_DEVTYPE_9M | 0x00000005 | 9M |
| SNSDK_DEVTYPE_E10 | 0x00000006 | E10 |
| SNSDK_DEVTYPE_2X | 0x00000007 | 2X |

### Error Code

| Member Variable | Macrodefined value | Meaning |
| --- | --- | --- |
| SNSDK_ERR_OK | 0x00000000 | Operation successful |
| SNSDK_ERR_NOTINITIALIZED | 0x00000001 | Uninitialized |
| SNSDK_ERR_INITIALIZEFIAILZED | 0x00000002 | Initialization failed |
| SNSDK_ERR_AlREADYINITIALIZED | 0x00000003 | Repeated initialization |
| SNSDK_ERR_SAVEFAILED_TYPEERROR | 0x00000004 | Save failed, format error |
| SNSDK_ERR_CHECKDEVICEFAILED_PLENOTRIGHT | 0x00000005 | Failed to connect device, incorrect ple |
| SNSDK_ERR_CHECKDEVICEFAILED_NODEVICEFOUND | 0x00000006 | Failed to connect device, no device found |
| SNSDK_ERR_OPENORCREATSLNFAILED | 0x00000007 | Failed to create or open project |
| SNSDK_ERR_ENTERSCANFAILED | 0x00000008 | Enter scan failed |
| SNSDK_ERR_SCANFAILED | 0x00000009 | Scanning failed |
| SNSDK_ERR_ENDSCANFAILED | 0x0000000A | Generation failed |
| SNSDK_ERR_MESHFAILED | 0x0000000B | Grid failure |
| SNSDK_ERR_EXITSCANFAILED | 0x0000000C | Exit scan failed |
| SNSDK_ERR_CANCELSCANFAILED | 0x0000000D | Cancel scan failed |
| SNSDK_ERR_SAVEFAILED | 0x0000000E | Save failed |
| SNSDK_ERR_CREATNEWPROJECTFAILED | 0x00000010 | New project failed |
| SNSDK_ERR_WORKRANGE | 0x00000011 | The working distance range does not meet the requirements |
| SNSDK_ERR_UNIMPLEMENTED | 0x00000012 | Function not implemented |
| SNSDK_ERR_NULLFUNCTION | 0x00000013 | Empty function |
| SNSDK_ERR_STARTPLUGIN_FAIL | 0x00000014 | Failed to launch plugin |
| SNSDK_ERR_NULLDATA | 0x00000015 | Empty data |
| SNSDK_ERR_NULLSERVICE | 0x00000016 | Empty service |
| SNSDK_ERR_OUTOFMEMORY | 0x00000017 | Insufficient memory |
| SNSDK_ERR_UNKNOWN | 0x00000018 | Unknown error |
| SNSDK_ERR_INVALIDINPUT | 0x00000019 | Illegal input |
| SNSDK_ERR_TOOFEW_SCANDATA | 0x0000001A | Scan data too little |
| SNSDK_ERR_OPTIMIZE_POINT_FAIL | 0x0000001B | Optimizing point cloud failed |
| SNSDK_ERR_PROCESSERROR | 0x10000000 | Process call error |

### Scanning Mode

| Member Variable | Macrodefined value | Meaning |
| --- | --- | --- |
| SNSDK_ERR_OK | 0x00000000 | Laser scanning |
| SNSDK_SCANMODE_LASER | 0x00000000 | Speckle scanning |
| SNSDK_SCANMODE_SPECKLE | 0x00000001 | Infrared scanning |
| SNSDK_SCANMODE_INFRARED | 0x00000002 | Laser scanning |

### Scanning Status

| Member Variable | Macrodefined value | Meaning |
| --- | --- | --- |
| SNSDK_PRE_SCAN | 0x00000000 | Before pre-scan |
| SNSDK_PRE_SCANNING | 0x00000001 | Pre-scan |
| SNSDK_SCAN | 0x00000002 | Scanning |
| SNSDK_PAUSED | 0x00000003 | Pause |
| SNSDK_SCAN_STOPED | 0x00000004 | Stop |

### Object Type

| Member Variable | Macrodefined value | Meaning |
| --- | --- | --- |
| SNSDK_SCANOBJECT_NORMAL | 0x00000000 | Ordinary objects |
| SNSDK_SCANOBJECT_REFLECT | 0x00000001 | Reflective objects |
| SNSDK_SCANOBJECT_BLACK | 0x00000002 | Black object |

# Interface Definitions for Handheld Scanner

This section outlines the essential functions for initializing and releasing device handles when working with handheld scanners. Ensuring proper management of these handles is crucial for the efficient and error-free operation of the scanner within your application.

## Device Handle

### Initialize device handle

| Function | Sn3DInitialize |
| --- | --- |
| Purpose | Initializes a handle for interacting with the handheld scanner. |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DInitialize( HSN3DSDKSERVICE* hService, int iDeviceType ); |
| Parameters | - hService: A pointer to an SDK service handle. This handle acts as a unique identifier for the session, analogous to a file handle in file operations. - iDeviceType: An integer specifying the type of device. For the specific values this parameter can take, refer to the documented device type definitions. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - Initialize only one device handle at a time per device type. - Release any previously initialized device handle using Sn3DRelease before initializing a new handle, even if it's for a different device type. |

### Release device handle

| Function | Sn3DRelease |
| --- | --- |
| Purpose | Releases the resources associated with a device handle, effectively closing it. |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DRelease ( HSN3DSDKSERVICE* pService ); |
| Parameters | - hService: A pointer to an SDK service handle. This handle acts as a unique identifier for the session, analogous to a file handle in file operations. - iDeviceType: An integer specifying the type of device. For the specific values this parameter can take, refer to the documented device type definitions. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - To maintain optimal resource management and system stability, release each device handle when it's no longer needed. - Follow a strict initialize-release cycle to avoid resource leaks and ensure that device handles are properly managed throughout the application lifecycle. |

## Callbacks

### Registering callbacks

| Function | Sn3DRegisterCallback |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DRegisterCallback( HSN3DSDKSERVICE pService, SDKCORE_CALLBACK pfCallBack, void* pUserData ) ; |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. pfCallBack: The system returns system call results and event notifications through this callback function, such as device disconnection, initialization completion, project creation failure, etc. Typedef void (CallBack * SDKCORECALLBACK) ( int iEventType, int iExtendData, void * pData, size_t stDataLen, void * pUserData ); - pUserData: [in] The last parameter of the pfCallBack function, which is returned to the user when the callback is triggered |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - After registering this callback function, the system will notify the registrant of the event and call result. The current version does not have anti registration or duplicate registration functions. |

### Callback response function

| Function | SDKCORE_CALLBACK |
| --- | --- |
| Purpose |  |
| Syntax | typedef void (CALLBACK *SDKCORE_CALLBACK)( int iEventType, int iExtendData, void* pData, size_t stDataLen, void* pUserData ); |
| Parameters | - iEventType: [in] Event type. - iExtendData: [in] Event Extended Data. - pData: [in] Event data - stDataLen: [in] Bytes of event data - pUserData: [out] User data passed in during registration |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - This function can only be called after registration, and the implementation code of the function needs to be maintained by the SDK caller themselves. - The callback function is executed by the internal working thread of the SDK, so attention should be paid to multi-threaded processing externally. - The callback function cannot call other SDK API functions. |

### Callback messages

FreeScan devices include FreeScan UE, FreeScan UE Pro, and FreeScan Combo devices. The message is provided in JSON string format

| JSON String | Meaning | Parameters and remarks |
| --- | --- | --- |
| {"cmd":"deviceStatus", "offline":false} | Device status | "Offline", true indicates the device is disconnected, false indicates the device is online |
| {"cmd":"addPrj", "prjInfo":{"prjPath":"d:/testdata/Project1.laser_prj_uep"}} | Add Project | The generation of the new project name has been completed. Record the project list. This list is required for the command to save the point cloud. Please refer to the commands supported by the FreeScan device |
| {"cmd":"newProjectFinish"} | New project created successfully | Note that this message will not bring back the specific project name, which needs to be saved in the "addPrj" message |
| {"cmd":"saveScanDataReturn"} | Data saved |  |
| {"cmd":"updateStateWhenStopFusion"} | Fusion completed | Please refer to the client user manual for the specific meaning of integration |
| {"cmd":"initalCalibParam"} | Initialization calibration completed |  |
| {"cmd":"showProgress", "data": { "title":"Now loading", "data":"30", "visible":true } } | Progress of certain tasks | "Data" is an object node that contains the following three attributes internally. "Title" represents progress prompt information, "data" represents specific progress values, and "visible" indicates whether the progress bar is visible. Note: Important functions cannot be determined by the progress bar, which is mainly used for real-time reporting of completion status |
| {"cmd":"hideProgress"} | The progress bar is hidden |  |
| {"cmd":"CalibHeightOKChanged", "okheight":"2" } | Number of completed calibration cells | The grid indicator here is the green grid on the interface, with numbering starting from 0 |
| { "cmd":"setGroupIndex", "index":"2" } | Calibration completed group number | The group here refers to the completion of a straight line calibration action, with each straight line containing a set of grids |
| {"method":"meshResult", "success":true } | Packaging results | "Success", true indicates successful encapsulation |
| {"method":"saveMeshFinished", "finish":true } | Save packaging results | "Finish", true indicates successful saving of the encapsulated file |
| {"cmd":"showMaximized"} | System initialization completed |  |
| {"cmd":"calibCalculateResult", "calibResult":true, "camErr":"0.018922"} | Calibration results | "CalibResult", true indicates successful calibration. "CamErr" represents calibration residual |
| {"cmd":"sdkScanPageSwitched"} | Switch to the scanning interface |  |
| {"cmd":"ScanModeChange", "scanMode": 0} | Scan state switching | "ScanMode", see scanning status |
| {"cmd":"returnScanSoftSetting", "enablePreScanFlg": true, "uep_changepoint_arrayvalue":"0.01, 0.02, 0.05, 0.2, 0.3"} | Obtaining configuration results | "EnablePreScanFlg", true indicates enabling pre scanning "Uep_changepoint.arrayvalue" represents a list of point spacing supported by UE Pro devices, in string format, separated by commas |
| {"cmd":"generatePointCloudResult", "result":0 } | Generate framework point/point cloud results | "Result", SNSDK_ ERR_ OK indicates success, otherwise it indicates the corresponding error code. Please refer to the error code definition |
| {"cmd":"cancelScanFinish"} | Cancel scan completed | That is to say, deleting scanned data is completed |
| {"cmd":"showProgress", "data": { "title":"Change Point Distance", "value":1 } } | Modification of scanning point spacing completed | This is a special application of progress messages |
| {"cmd":"createOrLoadCurProjectFinished", "PROJECT_TYPE":"0", "LASER_GRADE":"26", "MATERIAL_TYPE":"0", "HIGH_SPEED_MODE":"0", "STRONG_LIGHT_MODE":"0", "BRIGHTNESS_LEVEL":"2", } | Open Project Completed | This message obtains a series of engineering information. "PROJECT TYPE", "0" and "2" represent scanned point clouds/grids, while other values represent framework points. LASER_GRADE is the number of laser lines. For FreeScan devices, the recommended values are "26", "7", "5", and "1". "MATERIAL-TYPE" represents the type of scanned object, "0" represents a regular object, and "2" represents a reflective object. "HIGH SPEED-MODE", "1" represents high-speed scanning engineering, "0" represents normal scanning engineering. "STRING-LIGHT-MODE", "1" represents strong light scanning engineering, and "0" represents normal scanning engineering. BRIGHTNESS LEVEL, brightness level, corresponding to the [0,17] level on the scanning control panel |

If you want to receive message by ZeroMQ protocol directly, please refer to this product ZeroMQ protocol user manual. Here is a simple introduct: receive message by ZeroMQ protocol from port 11399.

## ZMQ

### Send command over ZMQ

| Function | Sn3DSendMessage |
| --- | --- |
| Purpose | Send commands to SDK |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DSendMessage( HSN3DSDKSERVICE pService, const char* szVersion, const char* szMsgType, const char* szZMQTopic, const char* pMsg, size_t stLen, char* pResult, size_t* pstResultLen ); |
| Parameters | - hService: [in] Device handle Sn3DInitialize returns the device handle. - szVersion: [in] Protocol version number, currently "2.0". - szZMQTopic: [in] The theme of ZMQ is currently not restricted. Simply pass in a non empty string. For special cases, please refer to the instructions in the commands supported by FreeScan devicesstLen. - PMsg: [in] The command data to be sent, in JSON format string, containing commands that need to be processed by the SDK - StLen: [in] Byte length of pMsg - pResult: [in] The text string returned by the SDK, which only represents that the SDK has received the sent message. The specific business result is returned through a callback function. Please refer to registering a callback function for the callback function. - pstResultLen: [in/out] The byte length of pResult. When input, it represents the number of bytes in the pResult buffer opened by the caller, and when output, it represents the actual number of bytes returned by the SDK. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - Prepare 256 bytes for the current version of the pResult buffer. - pMsg is in JSON format and encoded in UTF-8 format, so if you want to use Chinese, please ensure that Chinese characters are correctly encoded. |

## Supported command

FreeScan devices include FreeScan UE, FreeScan UE Pro, and FreeScan Combo devices. The message is provided in JSON string format

| JSON String | Meaning | Parameters and remarks |
| --- | --- | --- |
| {"cmd":"new", "mode":0, "fileDir":"D:/myproject"} | Create new project | "Mode" represents scanning mode, referring to scanning mode |
|  |  | The folder where the "fileDir" new project is located |
| {"cmd":"openProject", "ScanType":"LASER", "fileDir":"D:/myproject", "fileName":"5.sln_laser_uep"} | Open Project | "ScanType" represents the scanning mode, represented by a string, "LASER" represents laser engineering, and "IR" represents infrared engineering |
|  |  | The folder where the "fileDir" project is located |
|  |  | "FileName" project file name |
| {"cmd":"scan", "method":"scanCtrl"} | Start scanning | If the pre scan function is enabled, this command can indicate the start of pre scan |
| {"cmd":"pause", "method":"scanCtrl"} | Pause scanning |  |
| {"cmd":"delete", "method":"scanCtrl"} | Delete scanned data | Clear the currently scanned data |
| {"cmd":"stop", "method":"scanCtrl"} | End scan | Scan point cloud mode, this function is the same as pausing scanning, but in scanning frame point mode, this function also indicates ending scanning and optimizing results |
| {"cmd", "saveScanDataToSrvCmd"},{"prjNameList", [“project1”]},{"saveFolder", “D:/pointcloud”}, {"saveAscFile",true},{"saveP3File", false},{"saveTbFile", false} | Save Point Cloud | "PrjNameList" is the name of the project list, which can be obtained based on the callback message after creating the project.Please refer to the messages supported by FreeScan devices |
|  |  | The folder where "saveFolder" saves data |
|  |  | Does "saveAscFile" save an asc format file |
|  |  | Does "saveP3File" save a p3 format file |
|  |  | Does "saveTbFile" save the tb format file |
| {“value”:true} | Prepare packaging | The topic of this command must be "meshPostProcess" |
|  |  | "Value" true indicates optimized packaging, otherwise it indicates non optimized packaging |
| {"cmd": "apply", "method":"apply", "fileter_level": 5, "remove_small_floating_parts":true, "max_trianlges_ischeck": true, "max_trianlges_value":-1, "fill_small_hole_ischeck", true, "fill_small_hole_value", -1, "remove_spike_ischeck", true, "marker_hole_filling_ischeck", true, "isRememberChoice", false | Start packaging | "Fileter_level" filtering level |
|  |  | "Remove_small floating parts", true to delete small miscellaneous pieces, false to not delete |
|  |  | "Max_trianlges_ischeck", true indicates enabling the maximum number of triangular patches limit |
|  |  | "Max_trianlges_value" refers to the maximum number of triangular patches, where -1 indicates no limit |
|  |  | "Fill_small_hole_ischeck", true indicates filling in a small hole |
|  |  | "Fill_small_hole_value" is the circumference of the small hole. If it exceeds this circumference, it is considered a large hole and will not be filled. -1 indicates that the small hole will not be filled |
|  |  | "Remove_spike_ischeck", true indicates removing small spikes |
|  |  | "Marker_holeunfilling_ischeck", true indicates filling in the marker hole |
|  |  | "IsRememberChoice", true means remember to choose. SDK usually specifies parameters every time, so this option is meaningless for SDK |
| {"cmd": "undoMesh"} | Cancel packaging | Cancel the packaging effect of this time. If you are not satisfied with the packaging effect and want to modify parameters to re encapsulate, you can call this interface |
| {"cmd": "confirmMesh"} | Confirm packaging |  |
| {"fileName":"mymesh", "folderPath":"d:/test2”, "format":"001000000", "method":"save", "postpageVisible":1, "saveAscFile":0, "saveStlFile":1, "saveObjFile":0, "savePlyFile":0, "saveP3File":0, "save3MfFile":0, "saveTxtFile":0, "saveCsvFile":0} | Save Encapsulation | "FileName" saves the file name |
|  |  | FolderPath is the folder path where the file is saved |
|  |  | "Format" file type (customizable, consistent with the following flag bits such as asc) |
|  |  | Save AscFile, corresponding to format [0] |
|  |  | "SaveStlFile" saves the Stl file, corresponding to format [2] |
|  |  | "SaveObjFile" saves the Obj file, corresponding to format [3] |
|  |  | Save PlyFile, corresponding to format [4] |
|  |  | "SaveP3File" saves the P3 file, corresponding to format [5] |
|  |  | "Save3MfFile" saves a 3Mf file, corresponding to format [0] |
|  |  | "SaveTxtFile" saves the Txt file, corresponding to format [6] |
|  |  | Save CsvFile, corresponding to format [7] |
| {"cmd": "setLaserGrade2", "index": 0} | Set laser line type | "Index" needs to distinguish devices. For UE Pro devices, 0 represents 26 lines, 1 represents 5 lines, and 2 represents single line.For Combo devices, 0 represents 26 lines, 1 represents 7 lines, and 2 represents single line |
| {"cmd": "setScanObject2", "index": 0} | Set scanning object type | "Index", 0 represents regular equipment, 1 represents reflective equipment, and 2 represents black equipment.Attention: 2 is about to be abandoned |
| {"cmd": "changeBrightStep2", "index": 0} | Set the brightness level of scanning point clouds | The brightness level of "index" corresponds to the position of the brightness slider on the client scanning control panel |
| {"cmd": "changeBrightRadio2", "index": 0} | Set frame points/photogrammetric brightness levels | "Index", 0 represents low brightness, 1 represents medium brightness, and 2 represents high brightness, corresponding to the brightness option radio box on the client scanning control panel |
| {"cmd": "setScanMode2", "ScanMode": 0} | Set scanning mode | Combo device specific interface |
| {"cmd": "loadFramework", "FileName": "d:/test/fram.p3”} | ScanMode, SNSDK_SCANMODE_ LASER stands for laser scanning, SNSDK_ SCANMODE_ INFRARED stands for infrared scanning, reference scanning mode |  |
| {"cmd": "setProjectType2", "index": 0} | Load framework points | The "FileName" framework points to a file name, usually a p3 file |
| {"cmd": "setPointDist2", "index": 2} | Set scan item type | "Index", 0 represents the scanning frame point, 1 represents the scanning grid, and 2 represents photogrammetry |
| {"cmd": "setIsHighSpeed2", "high": true} | Set point spacing | "Index" corresponds to the position of the point distance on the client scanning control panel |
| {"cmd": "setIsStrongLight2", "strong": true} | Set whether to enable high-speed mode | "High", true indicates enabling high-speed mode |
| {"cmd": "setIsQualityColor2", "color": true} | Set whether to enable strong light mode | "High", true indicates turning on strong light mode |
| {"cmd": "setIsIncFramework2", "increase": true} | Set whether to enable color quality indicator | "Color", true indicates enabling color quality indicator |
| {"cmd": "enterScanPage2"} | Set whether to enable incremental framework points | "Increase", true indicates enabling the incremental framework point |
| {"cmd": "setCalibMode2", "mode":0} | Enter the scanning page | FreeScan UE/UE Pro devices correspond to the new/open engineering interface, and Combo devices correspond to the laser/infrared interface |
| {"cmd": "loadScanSoftSetting"} | Enter the calibration page | "Mode", 0 represents factory calibration, and 1 represents quick calibration. Usually only quick calibration is required |
| {"cmd": "setRulerFileList2", "list":["d:/ruler/931-932.pgf", "777-778.pgf"]} | Obtain scanning software configuration information | The callback message that passes the configuration result is returned. Please refer to the messages supported by FreeScan devices |

If you wang to send command by ZeroMQ protocol directly, please refer to this product ZeroMQ protocol user manual. Here is a simple introduct: send message by ZeroMQ protocol to port 11398.

## Calibration Interface

### Set calibration mode

| Function | Sn3DSetCalibMode |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DSetCalibMode( HSN3DSDKSERVICE pService, int iMode ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. - iMode: [in] Calibration mode 0 represents fast calibration, 1 represents factory calibration. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - The actual encoding of iMode can consider using SNSDK_CALIBCODE_ QUICK and SNSDK_ CALIBCODE_ FACTORY macro. |

### Start calibration

| Function | Sn3DCaliSnapImage |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DCaliSnapImage( HSN3DSDKSERVICE pService ) |
| Parameters | - pService: [in] Device handle Sn3DInitialize returns the device handle |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - After starting calibration, the SDK will send a message to inform the caller of the calibration status, refer to the messages supported by FreeScan devices. |

### End calibration

| Function | Sn3DStopCalib |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DStopCalib( HSN3DSDKSERVICE pService, int iMode ) |
| Parameters | - pService: [in] Device handle Sn3DInitialize returns the device handle. - iMode: [in] Calibration mode 0 represents fast calibration, 1 represents factory calibration. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - The actual encoding of iMode can consider using SNSDK_ CALIBCODE_ QUICK and SNSDK_ CALIBCODE_ FACTORY macro. The meaning of ending calibration is to terminate the current calibration, and the SDK is still in calibration initialization mode. |

## Scanning Interface

### Create new project

| Function | Sn3DNewProject |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DNewProject( HSN3DSDKSERVICE pService, LPNewProject newProject ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. - newProject: [in] Control structure for new construction projects |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - For FreeScan, only szSlnDirPath and iScanMode are used.Where szSlnDirPath represents the folder where the new project is located. IScanMode can access SNSDK_ SCANMODE_ LASER (laser scanning), SNSDK_ SCANMODE_ SPECKLE (speckle scanning), SNSDK_ SCANMODE_ INFRARED (infrared scanning). |

### Set scanning parameters

| Function | Sn3DSetScanPars |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DSetScanPars( HSN3DSDKSERVICE pService, LPScanPars scanPars ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. - scanPars: [in] Scanning parameter scanning parameter structure pointer, the structure meaning refers to ScanPars scanning parameter structure. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - For FreeScan devices, only scanPointCloud, scanMarkers, and scanPhotographic are used.FreeScan uses other interfaces to implement the functions of other members. |

### Set laser line level

| Function | Sn3DSetLaserGrade |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DSetLaserGrade( HSN3DSDKSERVICE pService, int index ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. - index: [in] Laser line level index.For UEPro devices, 0 represents 26 lines, 1 represents 5 lines, and 2 represents 1 line, corresponding to the client interface. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - For different devices, the level index has different meanings, but this index corresponds to the interface of the client software. |

### Set whether to add frame points

| Function | Sn3DSetIsIncFramework |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DSetIsIncFramework( HSN3DSDKSERVICE pService, bool bInc ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. - bInc: [in] Incremental framework point switch, true indicates enabling incremental framework points, false indicates disabling incremental framework points. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks |  |

### Start scanning

| Function | Sn3DStartScan |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DStartScan( HSN3DSDKSERVICE pService ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - For FreeScan, depending on the client's configuration, if pre scanning is enabled, then starting pre scanning is also this method. |

### Pause scanning

| Function | Sn3DPauseScan |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_ API int WINAPI Sn3DPuseScan（ HSN3DSDKSERVICE pService ）; |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - For FreeScanUEPro devices, pausing scanning for seconds in point cloud mode is equivalent to ending scanning (non optimized). |

### Cancel scanning

| Function | Sn3DCancelScan |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DCancelScan( HSN3DSDKSERVICE pService ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - Canceling the scan will also delete the existing scan data. It is recommended to pop up a prompt box before calling to remind the customer. |

### End scanning

| Function | Sn3DEndScan |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DEndScan( HSN3DSDKSERVICE pService ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - This method has overloaded versions corresponding to other devices, and only the version used by FreeScan devices is introduced here. |

### Mesh

| Function | Sn3DMesh |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DMesh( HSN3DSDKSERVICE pService, LPFreeScanMeshPar pParam ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. - pParam: [in] Packaging structure refer to FreeScanMeshPar packaging parameter structure |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks |  |

### Confirm mesh

| Function | Sn3DConfirmMesh |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DConfirmMesh( HSN3DSDKSERVICE pService ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks |  |

### Save mesh

| Function | Sn3DSaveMesh |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DSaveMesh( HSN3DSDKSERVICE pService, LPFreeSaveMeshPar pParam ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. - pParam: [in] Save the format structure, refer to FreeSaveMeshPar Save Encapsulation Structure |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks |  |

### Set scan mode

| Function | Sn3DSetScanMode |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DSetScanMode( HSN3DSDKSERVICE pService, int iMode ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. - pParam: [in] Save the format structure, refer to FreeSaveMeshPar Save Encapsulation Structure |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks |  |

### Enter scan page

| Function | Sn3DEnterScanPage |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_ API int WINAPI Sn3DEnterScanPage（ HSN3DSDKSERVICE pService ）; |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks |  |

### Enter scan mode page

| Function | Sn3DEnterScanModePage |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_ API int WINAPI Sn3DEnterScanModePage（ HSN3DSDKSERVICE pService ）; |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - For FreeScan, the corresponding client page is the "Laser/Infrared" engineering page.. |

### Set brightness

| Function | Sn3DChangeBrightStep |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_ API int WINAPI Sn3DChangeBrightStep（ HSN3DSDKSERVICE pService, Int iIndex ）; |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. - index: [in] Brightness level index.Different devices may vary. For FreeScanUE Pro devices, there are a total of 18 levels, and the values that can be passed in are 0-17. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks |  |

### Select high-speed scanning mode

| Function | Sn3DSetIsHighSpeed |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DSetIsHighSpeed( HSN3DSDKSERVICE pService, bool bIsHigh ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. - bIsHigh: [in] High speed mode switch, true to enable high-speed scanning mode, false to disable high-speed scanning mode. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks |  |

### Set scan object

| Function | Sn3DSetScanObject |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DSetScanObject( HSN3DSDKSERVICE pService, int iIndex ); |
| Parameters | - handle: [in] Device handle Sn3DInitialize returns the device handle. - index: [in] Object type index, which may vary for different devices and scanning modes. For FreeScanUE Pro devices, 0 represents a regular object and 1 represents a reflective object.Refer to the object type macro definition. |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - Some modes use 2 to represent black objects. |

## Get Data

### Register scene data callback

| Function | Sn3DRegisterSceneDataCallback |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DRegisterSceneDataCallback( Sn3DSceneDataCallBack pfCallBack, void* pUserData ); |
| Parameters | - Sn3DSceneDataCallBack: [in] Scene data callback - pUserData: [out] User data passed in during registration |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - This function is used to register the scene data update callback function pfCallback; - After scanning the 3D data, the callback function pfCallback will be continuously triggered; - It is recommended to call when scanning is turned on. |

### Get scene data

| Function | Sn3DGetSceneData |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DGetSceneData( LPSN3D_SCENE_DATA sceneData ); |
| Parameters | - GetSceneData: Scene data |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - This function is used to obtain scene data; - In each callback, it needs to be called twice. The first time is to allocate memory, and the second time can retrieve scene data from shared memory; - SDKs transfer scene data such as point cloud grids through shared memory.Allocate memory size as the length of the structure array * sizeof (structure size); - Please note that allocating memory also requires corresponding memory release to prevent memory leakage; - During 5 scans, the current landmark point pCurrentMarkers, overall landmark point pWholeMarkers, current point cloud pCurrentCloud, overall point cloud pWholeCloud, rotation matrix translation vector aCurrentCloudRt, and incremental mesh mIncreaseMesh will be transmitted. After pausing the scan, the overall mesh mMesh will be transmitted. After packaging is completed, the packaging mesh mMesh will be transmitted; - Example: void CALLBACK SDKWrapperSceneDataCallBack(void* pUserData) { SN3D_SCENE_DATA sceneData; memset(&sceneData, 0, sizeof(SN3D_SCENE_DATA)); getSceneData(&sceneData);// First call, no data transfer, notification about data size to be sent soon // Allocate memory**************************** sceneData.pWholeMarkers = new SN3D_MARKER[sceneData.nWholeMarkersCount]; sceneData.pCurrentMarkers = new SN3D_MARKER[sceneData.nCurrentMarkersCount]; sceneData.pCurrentCloud.pPointCoords = new SN3D_POINT_DATA[sceneData.nCurrentCloudCount]; sceneData.pCurrentCloud.pPointNormals = new SN3D_POINT_DATA[sceneData.nCurrentCloudCount]; sceneData.pCurrentCloud.pPointColors = new SN3D_POINT_DATA[sceneData.nCurrentCloudCount]; sceneData.pWholeCloud.pPointCoords = new SN3D_POINT_DATA[sceneData.nWholeCloudCount]; sceneData.pWholeCloud.pPointNormals = new SN3D_POINT_DATA[sceneData.nWholeCloudCount]; sceneData.pWholeCloud.pPointColors = new SN3D_POINT_DATA[sceneData.nWholeCloudCount]; sceneData.mMesh.pMeshPoint = new SN3D_POINT_DATA[sceneData.mMesh.nMeshpointCount]; sceneData.mMesh.pMeshNormal = new SN3D_POINT_DATA[sceneData.mMesh.nMeshNormalCount]; sceneData.mMesh.pMeshTriFaceId = new SN3D_TRI_FACE[sceneData.mMesh.nMeshTriFaceIdCount]; sceneData.mMesh.pMeshTextureId = new SN3D_TRI_FACE[sceneData.mMesh.nMeshTextureIdCount]; sceneData.mMesh.pTextureUV = new SN3D_UVCOORD[sceneData.mMesh.nTextureUVCount]; size_t imageSize = sceneData.mMesh.mImage.width*sceneData.mMesh.mImage.height*sceneData.mMesh.mImage.channel; sceneData.mMesh.mImage.data = new unsigned char[imageSize]; sceneData.mIncreaseMesh.pVertex = new SN3D_POINT_DATA[sceneData.mIncreaseMesh.nVertexCount]; sceneData.mIncreaseMesh.pMeshTriFaceId = new SN3D_TRI_FACE[sceneData.mIncreaseMesh.nMeshTriFaceIdCount]; sceneData.mIncreaseMesh.pWeightSet = new float[sceneData.mIncreaseMesh.nVertexCount]; //**************************** getSceneData(&sceneData);// Second call, transfer data // Release memory**************************** delete sceneData.pWholeMarkers ; delete sceneData.pCurrentMarkers ; delete sceneData.pCurrentCloud.pPointCoords; delete sceneData.pCurrentCloud.pPointNormals; delete sceneData.pCurrentCloud.pPointColors; delete sceneData.pWholeCloud.pPointCoords; delete sceneData.pWholeCloud.pPointNormals; delete sceneData.pWholeCloud.pPointColors; delete sceneData.mMesh.pMeshPoint ; delete sceneData.mMesh.pMeshNormal ; delete sceneData.mMesh.pMeshTriFaceId; delete sceneData.mMesh.pMeshTextureId; delete sceneData.mMesh.pTextureUV; delete sceneData.mMesh.mImage.data; delete sceneData.mIncreaseMesh.pVertex ; delete sceneData.mIncreaseMesh.pMeshTriFaceId; delete sceneData.mIncreaseMesh.pWeightSet; //**************************** } registerSceneDataCallback(SDKWrapperSceneDataCallBack, this);// Callbacks |

### Video data analysis

| Function | Sn3DInstallGetImagasCallBack |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DInstallGetImagasCallBack( Sn3DGetImagesCallBack imageCallback, void* owner ); |
| Parameters | - imageCallback: [in] Video callback function - owner: [in] class this pointer |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - Register the video callback function imageCallback; - It is recommended to register a callback after scanning is enabled; |

### Start process video data

| Function | Sn3DStartProcessVideoData |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DStartProcessVideoData( ); |
| Parameters |  |
| Return Values | Returns an integer code indicating success or providing an error code. Consult the documentation for error code meanings. |
| Remarks | - Start the video stream and successfully return 0; |

### Stop process video data

| Function | Sn3DStopProcessVideoData |
| --- | --- |
| Purpose |  |
| Syntax | SDKCWRAPPER_API int WINAPI Sn3DStopProcessVideoData( ); |
| Parameters |  |
| Return Values | - The video stream ends and returns 0 successfully; - Example: #include<opencv2/opencv.hpp> void CALLBACK SDKWrapperVideoCallback(int camId, const unsigned char * data, int width, int height, int channel, void* pUserData) { // Analyze the current frame of the video stream, where data is the current frame data pointer, width is the image width, height is the image height, and channel is the image channel cv::Mat image; if (channel == 3)cv::Mat img(height, width, CV_8UC3, data); else if (channel == 1)cv::Mat img(height, width, CV_8UC1, data); if (camId == 0)cv::imshow("leftImage", img);// Camid=0 is the left image else if (camId == 1)cv::imshow("RightImage", img);// Camid=1 is the right image cv::waitKey(0); return; } installGetImagasCallBack(SDKWrapperVideoCallback, this);// Register video stream callback Sn3DStartProcessVideoData();//Start video streaming //... Sn3DStopProcessVideoData();//End video stream |

|  | NOTICE There are many devices supported by the 3D SDK 2.0, and there are also some differences in the interfaces of different devices. If there are any inconsistencies in the documentation and demo code, please refer to the demo code for accuracy. SDK2.0 only supports English for operations involving file names due to functional requirements. Subsequent versions will include support for different languages. If you have any questions, please contact our company: support@shining3d.com |
| --- | --- |