SDK 2.0

User manual

Project Name: SDK2.0

Project number:

Responsible department: 3D Digital R&D Department

Product Manager: Yin Andong

catalogue

Revision Record 	3

SDK call process 	4

Data Structure and Macro Definition 	5

data structure 	5

Macro Definition 	9

Definition of handheld scanning interface 	11

Open device handle	11

Close device handle 	11

Registering Callbacks 	12

Callback response function 	12

Messages supported by FreeScan devices 	13

dispatch orders 	15

The commands supported by FreeScan devices 	16

Set calibration mode 	19

Start calibration 	19

End calibration 	20

New construction project 	20

Set scanning parameters 	21

Set laser line level 	21

Set whether to increment frame points 	22

Start scanning 	22

Pause scanning 	23

Cancel Scan	23

End scan 	24

Application encapsulation 	24

Confirm packaging 	25

Save Encapsulation 	25

Set scanning mode 	25

Enter the scanning page 	26

Enter the scanning mode page 	26

Set Brightness Level Page 	27

Set whether to set high-speed scanning mode 	27

Set scanning object type 	28

Shared memory parsing 	28

Video data analysis 	30

matters needing attention	32

# Revision Record

| version | Modify content | date |
| --- | --- | --- |
| 0.0.2 | Add a chapter on calling processes | January 4, 2024 |
| 0.0.1 | First draft generation | December 28, 2023 |

# SDK call process

Firstly, please complete the environment configuration according to XL-QER-RD-012 SDK 2.0 Development Environment Configuration SDK 2.0. docx. Then call the interface function defined in SDKCRapper. h, and process the next application logic according to the return value and the result callback function. Please refer to the definition of the handheld scanning interface.

Call process diagram

Callback (command parsing) diagram

Note: Command sending and callback are two independent logical processes that do not interfere with each other. This process is similar to common network programs, where the sending module is only responsible for sending and informing whether the sending result is successful or failed. The receiving (callback) process is only responsible for bringing back the current result of the system. For this SDK, callbacks may include device availability, execution results of a certain instruction, available system memory, etc.

# Data Structure and Macro Definition

  - data structure

| SN3D_ SCENE_ DATA member variables | meaning |
| --- | --- |
| NCurrentMarkerCount | Current number of landmark points |
| NWholeMarkerCount | Number of overall landmark points |
| NCurrentCloudCount | Current number of landmark points |
| NWholeCloudCount | Number of overall landmark points |
| PCurrentMarkers | Pointer to the current set of flag points, type SN3D_ Marker |
| PWholeMarkers | Pointer to the overall set of flag points, type SN3D_ Marker |
| PCurrentCloud | Pointer to the current point cloud array, type SN3D_ CLOUDPOINT |
| PWholeCloud | Pointer to the overall point cloud array, type SN3D_ CLOUDPOINT |
| ACurrentCloudRt | Rotation matrix and translation vector, type RT structure |
| MMesh | Overall grid, type SN3D_ MESH_ DATA overall grid structure |
| MIncrementMesh | Incremental grid, type SN3D_ INCREASE_ MESH_ DATA incremental grid structure |

| SN3D_ Marker flag point member variable | meaning |
| --- | --- |
| MCoords | Flag point normal vector array, with a length of 3 float types |
| MNormals | Flag point normal vector array, with a length of 3 float types |
| NId | NId, number |
| NDimeter | Float type, diameter |
| NCodeId | Int type, encoding point ID, ID less than 0 is a regular flag point |
| BIsDelete | Bool type, do you want to delete or change flag points |

| SN3D_ CLOUDPOINT point cloud structure member variable | meaning |
| --- | --- |
| PPointCoords | Point cloud coordinate array, SN3D_ Point_ DATA type |
| PPointNormals | Point cloud normal vector array, SN3D_ Point_ DATA type |
| PPointColors | Point cloud texture color array, SN3D_ Point_ DATA type |

| SN3D_ Point_ DATA point cloud data structure member variables | meaning |
| --- | --- |
| NX | Float type |
| NY | Float type |
| NZ | Float type |

| SN3D_ SCANNER_ RT structure member variables |  |
| --- | --- |
| ARot | Rotation matrix array, with a length of 9 float types |
| ATrans | Translation vector array, with a length of 3 float types |

| SN3D_ MESH_ DATA overall grid structure member variables |  |
| --- | --- |
| NMeshpointCount | Vertex array length |
| NMeshNormalCount | Normal vector array length |
| NMeshTriFaceIdCount | Point cloud index array length |
| NMeshTextureIdCount | Texture index array length |
| NTextureUVCount | Texture UV coordinate array length |
| PMeshPoint | Vertex array, each element is SN3D_ Point_ DATA type, representing point cloud (x, y, z) coordinates |
| PMeshNormal | Normal vector array, each element is SN3D_ Point_ DATA type, representing vectors perpendicular to the patch |
| PMeshTriFaceId | Point cloud index array, each element is SN3D_ TRI_ FACE type, representing the index of 3 vertices |
| PMeshTextureId | Texture index array, each element is SN3D_ TRI_ FACE type |
| PTextureUV | Texture UV coordinate array, each element is SN3D_ UVCOORD type |
| MImage | Texture map, type SN3D_ IMAGE_ DATA |

| SN3D_ IMAGE_ DATA texture map structure member variables | meaning |
| --- | --- |
| Width | Int type, texture map width |
| Height | Int type, texture map height |
| Channel | Int type, texture map channel |
| Data | Unsigned char * type, texture map data pointer |

| SN3D_ TRI_ FACE index structure member variables | meaning |
| --- | --- |
| X | Int type, vertex x index |
| Y | Int type, vertex y index |
| Z | Int type, vertex z-index |

| SN3D_ UVCOORD structure member variables | meaning |
| --- | --- |
| Uu | Int type, u coordinate |
| Vv | Int type, v coordinate |

| SN3D_ INCREASE_ MESH_ DATA incremental grid structure member variables | meaning |
| --- | --- |
| NVertexCount | Int type, number of vertices |
| PVertex | Vertex array, each element is SN3D_ Point_ DATA type |
| NMeshTriFaceIdCount | Int type, temporarily reserved, but also requires memory allocation |
| PMeshTriFaceId | SN3D_ TRI_ FACE type, temporarily reserved, but also requires memory allocation |
| PWeightSet | Float * type, weight of each vertex, used for mass chromatography |

ScanPars scanning parameter structure

| Member variables | meaning |
| --- | --- |
| ScanPointCloud | Bool type, whether to scan point cloud/grid |
| ScanMarkers | Bool type, whether to scan frame points |
| ScanPhotographic | Bool type, whether photogrammetry |

FreeScanMeshPar package parameter structure

| Member variables | meaning |
| --- | --- |
| Fileter_ Level | Int type, filtering level |
| Remove_ Small_ Floating_ Parts | Int type, removed small debris size, meaning referring to client software |
| Max_ Trianlges_ Ischeck | Bool type, limit the number of triangular patches |
| Max_ Trianlges_ Value | Int type, maximum number of triangular patches |
| Fill_ Small_ Hole_ Ischeck | Bool type, whether to perform small hole filling detection |
| Fill_ Small_ Hole_ Value | Int type, the size of the small hole to be filled, with the meaning referring to the client software |
| Remove_ Spike_ Ischeck | Bool type, whether to remove small spikes |
| Marker_ Hole_ Filling_ Ischeck | Bool type, whether to detect small holes formed by marker points |
| IsRememberChoice | Bool type, not currently used |

FreeSaveMeshPar saves encapsulated structures

| Member variables | meaning |
| --- | --- |
| FileName | Const char * type, file name |
| FolderPath | Const char * type, folder name where the file is located |
| Format | Const char * type, format, for verification purposes, generates a string with the shape of "01000000" based on subsequent members such as saveAscFile |
| Method | Const char * type, simply pass in "save" |
| PostpageVisible | Bool type, for internal use, simply pass in true here |
| SaveAscFile | Int type, 1 represents saving the asc format, 0 represents not saving the asc format |
| SaveStlFile | Int type, 1 represents saving stl format, 0 represents not saving stl format |
| SaveObjFile | Int type, 1 represents saving obj format, 0 represents not saving obj format |
| SavePlyFile | Int type, 1 represents saving the ply format, 0 represents not saving the ply format |
| SaveP3File | Int type, 1 represents saving p3 format, 0 represents not saving p3 format |
| Save3MfFile | Int type, 1 represents saving 3mf format, 0 represents not saving 3mf format |
| SaveTxtFile | Int type, 1 represents saving txt format, 0 represents not saving txt format |
| SaveCsvFile | Int type, 1 represents saving CSV format, 0 represents not saving CSV format |

Macro Definition

    - Device Type Definition

| Equipment type | Macro Definition Value | meaning |
| --- | --- | --- |
| SNSDK_ DEVTYPE_ UE | 0x0000000 | UE devices |
| SNSDK_ DEVTYPE_ UEPRO | 0x00000001 | UE Pro device |
| SNSDK_ DEVTYPE_ COMBO | 0x00000002 | Combo equipment |
| SNSDK_ DEVTYPE_ TRAK | 0x00000003 | Trak device |
| SNSDK_ DEVTYPE_ 5M | 0x00000004 | 5M device |
| SNSDK_ DEVTYPE_ 9M | 0x00000005 | 9M equipment |
| SNSDK_ DEVTYPE_ E10 | 0x00000006 | E10 device |
| SNSDK_ DEVTYPE_ 2X | 0x00000007 | 2X device |

    - Error code definition

| Error code | Macro Definition Value | meaning |
| --- | --- | --- |
| SNSDK_ ERR_ OK | 0x0000000 | Operation successful |
| SNSDK_ ERR_ NOTITIALIZED | 0x00000001 | uninitialized |
| SNSDK_ ERR_ INITIALIZEFIAILED | 0x00000002 | initialization failed |
| SNSDK_ ERR_ AlREADYINITIALIZED | 0x00000003 | Repeated initialization |
| SNSDK_ ERR_ SAVEFAILED_ TYPEERROR | 0x00000004 | Save failed, format error |
| SNSDK_ ERR_ CHECKDECEFAILED_ PLENOTLIGHT | 0x00000005 | Failed to connect device, incorrect ple |
| SNSDK_ ERR_ CHECKDECEFAILED_ NODEVICEFOUND | 0x00000006 | Failed to connect device, no device found |
| SNSDK_ ERR_ OPENORCREATSLNFAILED | 0x00000007 | Failed to create or open project |
| SNSDK_ ERR_ ENTERSCANFAILED | 0x00000008 | Enter scan failed |
| SNSDK_ ERR_ SCANFAILED | 0x00000009 | Scanning failed |
| SNSDK_ ERR_ ENDSCANFAILED | 0x0000000A | Generation failed |
| SNSDK_ ERR_ MESHFAILED | 0x00000000B | Grid failure |
| SNSDK_ ERR_ EXITSCANFAILED | 0x00000000C | Exit scan failed |
| SNSDK_ ERR_ CANCELSCANFAILED | 0x00000000D | Cancel scan failed |
| SNSDK_ ERR_ SAVEFAILED | 0x0000000E | Save failed |
| SNSDK_ ERR_ CREATNEWPROJECTFAILED | 0x00000010 | New project failed |
| SNSDK_ ERR_ WORK RANGE | 0x00000011 | The working distance range does not meet the requirements |
| SNSDK_ ERR_ UNITED | 0x00000012 | Function not implemented |
| SNSDK_ ERR_ NULLFUSION | 0x0000013 | Empty function |
| SNSDK_ ERR_ STARTPLUGIN_ FAIL | 0x0000014 | Failed to launch plugin |
| SNSDK_ ERR_ NULLDATA | 0x00000015 | Empty data |
| SNSDK_ ERR_ NULLSERVICE | 0x00000016 | Empty service |
| SNSDK_ ERR_ OUTOFMEMORY | 0x00000017 | insufficient memory |
| SNSDK_ ERR_ UNKNOWN | 0x00000018 | unknown error |
| SNSDK_ ERR_ INVALIDINPUT | 0x00000019 | illegal input |
| SNSDK_ ERR_ TOOFEW_ SCANDATA | 0x0000001A | Scan data too little |
| SNSDK_ ERR_ Optimize_ Point_ FAIL | 0x0000001B | Optimizing point cloud failed |
| SNSDK_ ERR_ PROCESS | 0x10000000 | Process call error |

    - Scanning mode

| Scanning mode | Macro Definition Value | meaning |
| --- | --- | --- |
| SNSDK_ SCANMODE_ LASER | 0x0000000 | Laser scanning |
| SNSDK_ SCANMODE_ SPECKLE | 0x00000001 | Speckle scanning |
| SNSDK_ SCANMODE_ INFRARED | 0x00000002 | Infrared scanning |

    - Scanning status

| Scanning mode | Macro Definition Value | meaning |
| --- | --- | --- |
| SNSDK_ PRE_ SCAN | 0x0000000 | Before pre scanning |
| SNSDK_ PRE_ SCANNING | 0x00000001 | Pre scan |
| SNSDK_ SCAN | 0x00000002 | scanning |
| SNSDK_ PAUSED | 0x00000003 | suspend |
| SNSDK_ SCAN_ STOPED | 0x00000004 | cease |

    - Object Object Type

| Object Object Type | Macro Definition Value | meaning |
| --- | --- | --- |
| SNSDK_ SCANOBJECT_ NORMAL | 0x0000000 | Ordinary objects |
| SNSDK_ SCANOBJECT_ REFLECT | 0x00000001 | Reflective objects |
| SNSDK_ SCANOBJECT_ BLACK | 0x00000002 | Black object |

# Definition of handheld scanning interface

  - Open device handle

Sn3DInitialize

Initialize device handle

SDKCWRAPPER_ API int WINAPI Sn3DInitialize（ HSN3DSDKSERVICE * hService, Int iDeviceType ）;

Parameters

HService

[in] SDK call handle, similar to file handle, serves as the identifier for some column calls this time

IDeviceType

[in] Device Type Reference Device Type Definition

Return Values

Reference error code definition

Remarks

The function 1 initializes the device handle.

Only one type of device handle can be used at a time.

Sn3DRelease must be called to release a device handle of one type before Sn3DInitialize can be called again to initialize another type of device handle.

  - Close device handle

Sn3DRelease

Close device handle

SDKCWRAPPER_ API int WINAPI Sn3DRelease（ HSN3DSDKSERVICE * pService ）;

Parameters

PService

[in] Device handle Sn3DInitialize returns the device handle

Return Values

Reference error code definition

Remarks

The function of function 1 is to release device handle resources.

Only one type of device handle can be used at a time.

This function must be called to release a device handle of one type before Sn3DInitialize can be called again to initialize another type of device handle.

Registering Callbacks

Sn3DRegisterCallback

SDKCWRAPPER_ API int WINAPI Sn3DRegisterCallback（ HSN3DSDKSERVICE pService, SDKCORE_ CallBack pfCallBack, Void * pUserData ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

PfCallBack

The system returns system call results and event notifications through this callback function, such as device disconnection, initialization completion, project creation failure, etc.

Typedef void (CallBack * SDKCORECALLBACK) (int iEventType, int iExtendData, void * pData, size_t stDataLen, void * pUserData);

PUserData

[in] The last parameter of the pfCallBack function, which is returned to the user when the callback is triggered

Return Values

Reference error code definition

Remarks

After registering this callback function, the system will notify the registrant of the event and call result. The current version does not have anti registration or duplicate registration functions.

Callback response function

SDKCORE_ Callback

Typedef void (CallBack * SDKCORE.CALLBACK)（ Int iEventType, Int iExtendData, Void * pData, Size_ T stDataLen, Void * pUserData ）;

Parameters

IEventType

[in] Event type

IExtendData

[in] Event Extended Data

PData

[in] Event data

StDataLen

[in] Bytes of event data

PUserData

[out] User data passed in during registration

Return Values

Reference error code definition

Remarks

This function can only be called after registration, and the implementation code of the function needs to be maintained by the SDK caller themselves;

The callback function is executed by the internal working thread of the SDK, so attention should be paid to multi-threaded processing externally;

The callback function cannot call other SDK API functions.

Messages supported by FreeScan devices

FreeScan devices include FreeScan UE, FreeScan UE Pro, and FreeScan Combo devices. The message is provided in JSON string format.

| JSON string | meaning | Parameters and remarks |
| --- | --- | --- |
| {"cmd": "deviceStatus", "offline": false} | Device status | "Offline", true indicates the device is disconnected, false indicates the device is online |
| {"cmd": "addPrj", "prjInfo": {"prjPath": "d:/testdata/Project1. laser_prj_uep"} | Add Project | The generation of the new project name has been completed. Record the project list. This list is required for the command to save the point cloud. Please refer to the commands supported by the FreeScan device |
| {"cmd": "newProjectFinish"} | New project successful | Note that this message will not bring back the specific project name, which needs to be saved in the "addPrj" message |
| {"cmd": "saveScanDataReturn"} | Save data completed |  |
| {"cmd": "updateStateWhenStopFusion"} | Fusion completed | Please refer to the client user manual for the specific meaning of integration |
| {"cmd": "initalCalibParam"} | Initialization calibration completed |  |
| {"cmd": "showProgress", "Data": { "Title": "Now loading", "Data": "30", "Visible": true } } | Progress of certain tasks | "Data" is an object node that contains the following three attributes internally. "Title" represents progress prompt information, "data" represents specific progress values, and "visible" indicates whether the progress bar is visible. Note: Important functions cannot be determined by the progress bar, which is mainly used for real-time reporting of completion status |
| {"cmd": "hideProgress"} | The progress bar is hidden |  |
| {"cmd": "CalibHeightOKChanged", "Okheight": "2" } | Number of completed calibration cells | The grid indicator here is the green grid on the interface, with numbering starting from 0 |
| {"cmd": "setGroupIndex", "Index": "2" } | Calibration completed group number | The group here refers to the completion of a straight line calibration action, with each straight line containing a set of grids |
| {"method": "meshResult", "Success": true } | Packaging results | "Success", true indicates successful encapsulation |
| {"method": "saveMeshFinished", "Finish": true } | Save packaging results | "Finish", true indicates successful saving of the encapsulated file |
| {"cmd": "showMaximized"} | System initialization completed |  |
| {"cmd": "calibCalculateResult", "CalibResult": true, "CamErr": "0.018922"} | Calibration results | "CalibResult", true indicates successful calibration. "CamErr" represents calibration residual |
| {"cmd": "sdkScanPageSwitched"} | Switch to the scanning interface |  |
| {"cmd": "ScanModeChange", "ScanMode": 0} | Scan state switching | "ScanMode", see scanning status |
| {"cmd": "returnScanSoftSetting", "EnablePreScanFlg": true, "Uep_changepoint.arrayvalue": "0.01, 0.02, 0.05, 0.2, 0.3"} | Obtaining configuration results | "EnablePreScanFlg", true indicates enabling pre scanning "Uep_changepoint.arrayvalue" represents a list of point spacing supported by UE Pro devices, in string format, separated by commas |
| {"cmd": "generatePointCloudResult", "Result": 0 } | Generate framework point/point cloud results | "Result", SNSDK_ ERR_ OK indicates success, otherwise it indicates the corresponding error code. Please refer to the error code definition |
| {"cmd": "cancelScanFinish"} | Cancel scan completed | That is to say, deleting scanned data is completed |
| {"cmd": "showProgress", "Data": { "Title": "Change Point Distance", Value: 1 } } | Modification of scanning point spacing completed | This is a special application of progress messages |
| {"cmd": "createOrLoadCurProjectCompleted", "PROJECT TYPE": "0", "LASER_GRADE": "26", "Material-TYPE": "0", "HIGH SPEED-MODE": "0", "STRING-LIGHT-MODE": "0", "BRIGHTNESS LEVEL": "2", } | Open Project Completed | This message obtains a series of engineering information. "PROJECT TYPE", "0" and "2" represent scanned point clouds/grids, while other values represent framework points. LASER_GRADE is the number of laser lines. For FreeScan devices, the recommended values are "26", "7", "5", and "1". "MATERIAL-TYPE" represents the type of scanned object, "0" represents a regular object, and "2" represents a reflective object. "HIGH SPEED-MODE", "1" represents high-speed scanning engineering, "0" represents normal scanning engineering. "STRING-LIGHT-MODE", "1" represents strong light scanning engineering, and "0" represents normal scanning engineering. BRIGHTNESS LEVEL, brightness level, corresponding to the [0,17] level on the scanning control panel |
| { "cmd":"currentRTcmd", "rot0": 0.3785639731719602, "rot1": -0.25789540718875675, "rot2": -0.888920287296451, "rot3": 0.00612465152633862, "rot4": 0.961074718181977, "rot5": -0.27622069928793314, "rot6": 0.9255548643165032, "rot7": 0.09912287840039362, "rot8": 0.36540778332936547, "trans0": -181.08949382871026, "trans1": -16.99803930550829, "trans2": 150.74391990416274 } |  | Current point cloud RT. Rot0-rot9 is the rotate matrix. Trans0-trans2 is transfer vector. If you want to learn more details about RT, please refer to our client software user manual and computer graphics books. |

If you wang to receive message by ZeroMQ protocol directly, please refer to this product ZeroMQ protocol user manual. Here is a simple introduct: receive message by ZeroMQ protocol from port 11399.

  - dispatch commands

Sn3DSendMessage

Send commands to SDK

SDKCWRAPPER_ API int WINAPI Sn3DSendMessage（ HSN3DSDKSERVICE pService, Const char * szVersion, Const char * szMsgType, Const char * szZMQTopic, Const char * pMsg, Size_ T stLen, Char * pResult, Size_ T * pstResultLen ）;

Parameters

HService

[in] Device handle Sn3DInitialize returns the device handle

SzVersion

[in] Protocol version number, currently "2.0"

SzZMQTopic

[in] The theme of ZMQ is currently not restricted. Simply pass in a non empty string. For special cases, please refer to the instructions in the commands supported by FreeScan devices

PMsg

[in] The command data to be sent, in JSON format string, containing commands that need to be processed by the SDK

StLen

[in] Byte length of pMsg

PResult

[in] The text string returned by the SDK, which only represents that the SDK has received the sent message. The specific business result is returned through a callback function. Please refer to registering a callback function for the callback function.

PstResultLen

[in/out] The byte length of pResult. When input, it represents the number of bytes in the pResult buffer opened by the caller, and when output, it represents the actual number of bytes returned by the SDK.

Return Values

Reference error code definition

Remarks

Prepare 256 bytes for the current version of the pResult buffer.

2 pMsg is in JSON format and encoded in UTF-8 format, so if you want to use Chinese, please ensure that Chinese characters are correctly encoded.

  - The commands supported by FreeScan devices

FreeScan devices include FreeScan UE, FreeScan UE Pro, and FreeScan Combo devices. The command is sent as a JSON string.

| JSON string | meaning | Parameters and remarks |
| --- | --- | --- |
| {"cmd": "new", "mode": 0, "fileDir": "D:/myproject"} | New construction project | "Mode" represents scanning mode, referring to scanning mode |
|  |  | The folder where the "fileDir" new project is located |
| {"cmd": "openProject", "ScanType": "LASER", "fileDir": "D:/myproject", "FileName": "5. sln.laser_uep"} | Open Project | "ScanType" represents the scanning mode, represented by a string, "LASER" represents laser engineering, and "IR" represents infrared engineering |
|  |  | The folder where the "fileDir" project is located |
|  |  | "FileName" project file name |
| {"cmd": "scan", "method": "scanCtrl"} | Start scanning | If the pre scan function is enabled, this command can indicate the start of pre scan |
| {"cmd": "pause", "method": "scanCtrl"} | Pause scanning |  |
| {"cmd": "delete", "method": "scanCtrl"} | Delete scan | Clear the currently scanned data |
| {"cmd": "stop", "method": "scanCtrl"} | End scan | Scan point cloud mode, this function is the same as pausing scanning, but in scanning frame point mode, this function also indicates ending scanning and optimizing results |
| {"cmd", "saveScanDataToSrvCmd"}, {"prjNameList", ["project1"]}, {"saveFolder", "D:/pointcloud"}, {"saveAscFile", true}, {"saveP3File", false}, {"saveTbFile", false} | Save Point Cloud | "PrjNameList" is the name of the project list, which can be obtained based on the callback message after creating the project.Please refer to the messages supported by FreeScan devices |
|  |  | The folder where "saveFolder" saves data |
|  |  | Does "saveAscFile" save an asc format file |
|  |  | Does "saveP3File" save a p3 format file |
|  |  | Does "saveTbFile" save the tb format file |
| {"value": true} | Prepare packaging | The topic of this command must be "meshPostProcess" |
|  |  | "Value" true indicates optimized packaging, otherwise it indicates non optimized packaging |
| {"cmd": "apply", "Method": "apply", "Fileter_level": 5, "Remove_small floating parts": true, "Max_trianlges_ischeck": true, "Max_trianlges_value": -1, "Fill_small_hole_ischeck", true, "Fill_small_hole_value", -1, "Remove_spike_ischeck", true, "Marker_hole filling ischeck", true, "IsRememberChoice", false | Start packaging | "Fileter_level" filtering level |
|  |  | "Remove_small floating parts", true to delete small miscellaneous pieces, false to not delete |
|  |  | "Max_trianlges_ischeck", true indicates enabling the maximum number of triangular patches limit |
|  |  | "Max_trianlges_value" refers to the maximum number of triangular patches, where -1 indicates no limit |
|  |  | "Fill_small_hole_ischeck", true indicates filling in a small hole |
|  |  | "Fill_small_hole_value" is the circumference of the small hole. If it exceeds this circumference, it is considered a large hole and will not be filled. -1 indicates that the small hole will not be filled |
|  |  | "Remove_spike_ischeck", true indicates removing small spikes |
|  |  | "Marker_holeunfilling_ischeck", true indicates filling in the marker hole |
|  |  | "IsRememberChoice", true means remember to choose. SDK usually specifies parameters every time, so this option is meaningless for SDK |
| {"cmd": "undoMesh"} | Unpacking | Cancel the encapsulation effect this time. If you are not satisfied with the encapsulation effect and want to modify parameters to re encapsulate, you can call this interface |
| {"cmd": "confirmMesh"} | Confirm packaging |  |
| {"fileName": "mymesh", "FolderPath": "d:/test2", "Format": "001000000", "Method": "save", "PostpageVisible": 1, "SaveAscFile": 0, "SaveStlFile": 1, "SaveObjFile": 0, "SavePlyFile": 0, "SaveP3File": 0, "Save3MfFile": 0, "SaveTxtFile": 0, "SaveCsvFile": 0} | Save Encapsulation | "FileName" saves the file name |
|  |  | FolderPath is the folder path where the file is saved |
|  |  | "Format" file type (customizable, consistent with the following flag bits such as asc) |
|  |  | Save AscFile, corresponding to format [0] |
|  |  | "SaveStlFile" saves the Stl file, corresponding to format [2] |
|  |  | "SaveObjFile" saves the Obj file, corresponding to format [3] |
|  |  | Save PlyFile, corresponding to format [4] |
|  |  | "SaveP3File" saves the P3 file, corresponding to format [5] |
|  |  | "Save3MfFile" saves a 3Mf file, corresponding to format [0] |
|  |  | "SaveTxtFile" saves the Txt file, corresponding to format [6] |
|  |  | Save CsvFile, corresponding to format [7] |
| {"cmd": "setLaserGrade2", "Index": 0} | Set laser line type | "Index" needs to distinguish devices. For UE Pro devices, 0 represents 26 lines, 1 represents 5 lines, and 2 represents single line.For Combo devices, 0 represents 26 lines, 1 represents 7 lines, and 2 represents single line |
| {"cmd": "setScanObject 2", "Index": 0} | Set scanning object type | "Index", 0 represents regular equipment, 1 represents reflective equipment, and 2 represents black equipment.Attention: 2 is about to be abandoned |
| {"cmd": "changeBrightStep2", "Index": 0} | Set the brightness level of scanning point clouds | The brightness level of "index" corresponds to the position of the brightness slider on the client scanning control panel |
| {"cmd": "changeBrightRadio2", "Index": 0} | Set frame points/photogrammetric brightness levels | "Index", 0 represents low brightness, 1 represents medium brightness, and 2 represents high brightness, corresponding to the brightness option radio box on the client scanning control panel |
| {"cmd": "setScanMode2", ScanMode: 0} | Set scanning mode | Combo device specific interface ScanMode, SNSDK_SCANMODE_ LASER stands for laser scanning, SNSDK_ SCANMODE_ INFRARED stands for infrared scanning, reference scanning mode |
| {"cmd": "loadFramework", "FileName": "d:/test/fram. p3"} | Load framework points | The "FileName" framework points to a file name, usually a p3 file |
| {"cmd": "setProjectType2", "Index": 0} | Set scan item type | "Index", 0 represents the scanning frame point, 1 represents the scanning grid, and 2 represents photogrammetry |
| {"cmd": "setPointDist2", "Index": 2} | Set point spacing | "Index" corresponds to the position of the point distance on the client scanning control panel |
| {"cmd": "setIsHighSpeed2", "High": true} | Set whether to enable high-speed mode | "High", true indicates enabling high-speed mode |
| {"cmd": "setIsStrongLight2", "Strong": true} | Set whether to enable strong light mode | "High", true indicates turning on strong light mode |
| {"cmd": "setIsQualityColor2", "Color": true} | Set whether to enable color quality indicator | "Color", true indicates enabling color quality indicator |
| {"cmd": "setIsIncFramework2", "Increase": true} | Set whether to enable incremental framework points | "Increase", true indicates enabling the incremental framework point |
| {"cmd": "enterScanPage2"} | Enter the scanning page | FreeScan UE/UE Pro devices correspond to the new/open engineering interface, and Combo devices correspond to the laser/infrared interface |
| {"cmd": "setCalibMode2", "mode": 0} | Enter the calibration page | "Mode", 0 represents factory calibration, and 1 represents quick calibration. Usually only quick calibration is required |
| {"cmd": "loadScanSoftSetting"} | Obtain scanning software configuration information | The callback message that passes the configuration result is returned. Please refer to the messages supported by FreeScan devices |
| {"cmd": "setRulerFileList2", "list": ["d:/Ruler/931-932. pgf", "777-778. pgf"]} | Set ruler file | List of ruler files |

If you wang to send command by ZeroMQ protocol directly, please refer to this product ZeroMQ protocol user manual. Here is a simple introduct: send message by ZeroMQ protocol to port 11398.

Set calibration mode

Sn3DSetCalibMode

SDKCWRAPPER_ API int WINAPI Sn3DSetCalibMode（ HSN3DSDKSERVICE pService, Int iMode ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

IMode

[in] Calibration mode 0 represents fast calibration, 1 represents factory calibration

Return Values

Reference error code definition

Remarks

The actual encoding of iMode can consider using SNSDK_CALIBCODE_ QUICK and SNSDK_ CALIBCODE_ FACTORY macro.

Start calibration

Sn3DAliSnapImage

SDKCWRAPPER_ API int WINAPI Sn3DAliSnapImage（ HSN3DSDKSERVICE pService ）

Parameters

PService

[in] Device handle Sn3DInitialize returns the device handle

Return Values

Reference error code definition

Remarks

After starting calibration, the SDK will send a message to inform the caller of the calibration status, refer to the messages supported by FreeScan devices.

End calibration

Sn3DTopCalib

SDKCWRAPPER_ API int WINAPI Sn3DTopCalib（ HSN3DSDKSERVICE pService, Int iMode ）

Parameters

PService

[in] Device handle Sn3DInitialize returns the device handle

IMode

[in] Calibration mode 0 represents fast calibration, 1 represents factory calibration

Return Values

Reference error code definition

Remarks

The actual encoding of iMode can consider using SNSDK_ CALIBCODE_ QUICK and SNSDK_ CALIBCODE_ FACTORY macro. The meaning of ending calibration is to terminate the current calibration, and the SDK is still in calibration initialization mode.

New construction project

Sn3DNewProject

SDKCWRAPPER_ API int WINAPI Sn3DNewProject（ HSN3DSDKSERVICE pService, LPNewProject newProject ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

NewProject

[in] Control structure for new construction projects

For FreeScan, only szSlnDirPath and iScanMode are used.Where szSlnDirPath represents the folder where the new project is located. IScanMode can access SNSDK_ SCANMODE_ LASER (laser scanning), SNSDK_ SCANMODE_ SPECKLE (speckle scanning), SNSDK_ SCANMODE_ INFRARED (infrared scanning).

Return Values

Reference error code definition

Set scanning parameters

Sn3DSetScanPars

SDKCWRAPPER_ API int WINAPI Sn3DSetScanPars（ HSN3DSDKSERVICE pService, LPScanPars scanPars ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

ScanPars

[in] Scanning parameter scanning parameter structure pointer, the structure meaning refers to ScanPars scanning parameter structure.

Return Values

Reference error code definition

Remarks

For FreeScan devices, only scanPointCloud, scanMarkers, and scanPhotographic are used.FreeScan uses other interfaces to implement the functions of other members.

Set laser line level

Sn3DSetLaserGrade

SDKCWRAPPER_ API int WINAPI Sn3DSetLaserGrade（ HSN3DSDKSERVICE pService, Int index ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

Index

[in] Laser line level index.For UEPro devices, 0 represents 26 lines, 1 represents 5 lines, and 2 represents 1 line, corresponding to the client interface.

Return Values

Reference error code definition

Remarks

For different devices, the level index has different meanings, but this index corresponds to the interface of the client software.

Set whether to increment frame points

Sn3DSetIsIncFramework

SDKCWRAPPER_ API int WINAPI Sn3DSetIsIncFramework（ HSN3DSDKSERVICE pService, Bool bInc ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

BInc

[in] Incremental framework point switch, true indicates enabling incremental framework points, false indicates disabling incremental framework points.

Return Values

Reference error code definition

Start scanning

Sn3DStartScan

SDKCWRAPPER_ API int WINAPI Sn3DStartScan（ HSN3DSDKSERVICE pService ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

Return Values

Reference error code definition

Remarks

For FreeScan, depending on the client's configuration, if pre scanning is enabled, then starting pre scanning is also this method.

Pause scanning

Sn3DPuseScan

SDKCWRAPPER_ API int WINAPI Sn3DPuseScan（ HSN3DSDKSERVICE pService ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

Return Values

Reference error code definition

Remarks

For FreeScanUEPro devices, pausing scanning for seconds in point cloud mode is equivalent to ending scanning (non optimized).

Cancel Scan

Sn3DCancelScan

SDKCWRAPPER_ API int WINAPI Sn3DCancelScan（ HSN3DSDKSERVICE pService ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

Return Values

Reference error code definition

Remarks

Canceling the scan will also delete the existing scan data. It is recommended to pop up a prompt box before calling to remind the customer.

End scan

Sn3DEndScan

SDKCWRAPPER_ API int WINAPI Sn3DEndScan（ HSN3DSDKSERVICE pService ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

Return Values

Reference error code definition

Remarks

This method has overloaded versions corresponding to other devices, and only the version used by FreeScan devices is introduced here.

Application encapsulation

Sn3DMesh

SDKCWRAPPER_ API int WINAPI Sn3DMesh（ HSN3DSDKSERVICE pService, LPFreeScanMeshPar pParam ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

PParam

[in] Packaging structure refer to FreeScanMeshPar packaging parameter structure

Return Values

Reference error code definition

Confirm packaging

Sn3DConfirmMesh

SDKCWRAPPER_ API int WINAPI Sn3DConfirmMesh（ HSN3DSDKSERVICE pService ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

Return Values

Reference error code definition

Save Encapsulation

Sn3DSaveMesh

SDKCWRAPPER_ API int WINAPI Sn3DSaveMesh（ HSN3DSDKSERVICE pService, LPFreeSaveMeshPar pParam ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

PParam

[in] Save the format structure, refer to FreeSaveMeshPar Save Encapsulation Structure

Return Values

Reference error code definition

Set scanning mode

Sn3DSetScanMode

SDKCWRAPPER_ API int WINAPI Sn3DSetScanMode（ HSN3DSDKSERVICE pService, Int iMode ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

PParam

[in] iMode sets the scanning mode, refer to the scanning mode macro definition

Return Values

Reference error code definition

Enter the scanning page

Sn3DEnterScanPage

SDKCWRAPPER_ API int WINAPI Sn3DEnterScanPage（ HSN3DSDKSERVICE pService ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

Return Values

Reference error code definition

Remarks

For FreeScan, the corresponding client page is the "New/Open" project page.

Enter the scanning mode page

Sn3DEnterScanModePage

SDKCWRAPPER_ API int WINAPI Sn3DEnterScanModePage（ HSN3DSDKSERVICE pService ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

Return Values

Reference error code definition

Remarks

For FreeScan, the corresponding client page is the "Laser/Infrared" engineering page.

Set Brightness Level Page

Sn3DChangeBrightStep

SDKCWRAPPER_ API int WINAPI Sn3DChangeBrightStep（ HSN3DSDKSERVICE pService, Int iIndex ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

Index

[in] Brightness level index.Different devices may vary. For FreeScanUE Pro devices, there are a total of 18 levels, and the values that can be passed in are 0-17.

Return Values

Reference error code definition

Set whether to set high-speed scanning mode

Sn3DSetIsHighSpeed

SDKCWRAPPER_ API int WINAPI Sn3DSetIsHighSpeed（ HSN3DSDKSERVICE pService, Bool bIsHigh ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

BIsHigh

[in] High speed mode switch, true to enable high-speed scanning mode, false to disable high-speed scanning mode.

Return Values

Reference error code definition

Set scanning object type

Sn3DSetScanObject

SDKCWRAPPER_ API int WINAPI Sn3DSetScanObject（ HSN3DSDKSERVICE pService, Int iIndex ）;

Parameters

Handle

[in] Device handle Sn3DInitialize returns the device handle

IIndex

[in] Object type index, which may vary for different devices and scanning modes. For FreeScanUE Pro devices, 0 represents a regular object and 1 represents a reflective object.Refer to the object type macro definition.

Return Values

Reference error code definition

Remarks

Some modes use 2 to represent black objects.

# Shared memory parsing

SDKCWRAPPER_ API int WINAPI Sn3DRegisterSceneDataCallback（ Sn3DSceneDataCallBack pfCallBack, Void * pUserData ）;

Parameters

Sn3DSceneDataCallBack

[in] Scene data callback

PUserData

[out] User data passed in during registration

Return Values

Reference error code definition

Remarks

This function is used to register the scene data update callback function pfCallback;

After scanning the 3D data, the callback function pfCallback will be continuously triggered;

It is recommended to call when scanning is turned on.

SDKCWRAPPER_ API int WINAPI Sn3DGetSceneData（ LPSN3D_ SCENE_ DATA sceneData ）;

Parameters

SceneData

[out] Scene data

Return Values

Reference error code definition

Remarks

This function is used to obtain scene data;

In each callback, it needs to be called twice. The first time is to allocate memory, and the second time can retrieve scene data from shared memory;

3 SDKs transfer scene data such as point cloud grids through shared memory.Allocate memory size as the length of the structure array * sizeof (structure size);

4. Please note that allocating memory also requires corresponding memory release to prevent memory leakage;

During 5 scans, the current landmark point pCurrentMarkers, overall landmark point pWholeMarkers, current point cloud pCurrentCloud, overall point cloud pWholeCloud, rotation matrix translation vector aCurrentCloudRt, and incremental mesh mIncreaseMesh will be transmitted. After pausing the scan, the overall mesh mMesh will be transmitted. After packaging is completed, the packaging mesh mMesh will be transmitted;

Example: Void CallBack SDKWrapperSceneDataCallBack (void * pUserData){ SN3D_ SCENE_ DATA sceneData; Memset (&sceneData, 0, sizeof (SN3D-SCENEDATA)); GetSceneData (&sceneData)// First call, no data transfer, notification about data size to be sent soon //Allocate memory**************************** SceneData. pWholeMarkers=new SN3D_ Marker [sceneData. nwholeMarkerCount]; SceneData. pCurrentMarkers=new SN3D_ Marker [sceneData. ncurrentMarkerCount]; SceneData. pCurrentCloud. pPointCoords=new SN3D_ Point_ DATA [sceneData. ncurrentCloudCount]; SceneData. pCurrentCloud. pPointNormals=new SN3D_ Point_ DATA [sceneData. ncurrentCloudCount]; SceneData. pCurrentCloud. pPointColors=new SN3D_ Point_ DATA [sceneData. ncurrentCloudCount]; SceneData. pWholeCloud. pPointCoords=new SN3D_ Point_ DATA [sceneData. nwholeCloudCount]; SceneData. pWholeCloud. pPointNormals=new SN3D_ Point_ DATA [sceneData. nwholeCloudCount]; SceneData. pWholeCloud. pPointColors=new SN3D_ Point_ DATA [sceneData. nwholeCloudCount]; SceneData. mMesh. pMeshPoint=new SN3D_ Point_ DATA [sceneData. mMesh. nmeshpointCount]; SceneData. mMesh. pMeshNormal=new SN3D_ Point_ DATA [sceneData. mMesh. nMeshNormalCount]; SceneData. mMesh. pMeshTriFaceId=new SN3D_ TRI_ FACE [sceneData. mMesh. nMeshTriFaceIdCount]; SceneData. mMesh. pMeshTextureId=new SN3D_ TRI_ FACE [sceneData. mMesh. nMeshTextureIdCount]; SceneData. mMesh. pTextureUV=new SN3D_ UVCOORD [sceneData. mMesh. nTextureUVCount]; Size_ T imageSize=sceneData. mMesh. mImage. width * sceneData. mMesh. mImage. height * sceneData. mMesh. mImage. channel; SceneData. mMesh. mImage. data=new unsigned char [imageSize]; SceneData. mIncrementMesh. pVertex=new SN3D_ Point_ DATA [sceneData. mIncrementMesh. nvertexCount]; SceneData. mIncrementMesh. pMeshTriFaceId=new SN3D_ TRI_ FACE [sceneData. mIncrementMesh. nMeshTriFaceIdCount]; SceneData. mIncrementMesh. pWeightSet=new float [sceneData. mIncrementMesh. nVertexCount]; //**************************** GetSceneData (&sceneData)// Second call, transfer data //Release memory**************************** Delete sceneData.pWholeMarkers; Delete sceneData.pCurrentMarkers; Delete sceneData. pcurrentCloud. ppPointCoords; Delete sceneData. pcurrentCloud. pPointNormals; Delete sceneData. pCurrentCloud. pPointColors; Delete sceneData.pWholeCloud. pPointCoords; Delete sceneData. pWholeCloud. pPointNormals; Delete sceneData. pWholeCloud. pPointColors; Delete sceneData. mMesh. pMeshPoint; Delete sceneData. mMesh. pMesh Normal; Delete sceneData. mMesh. pMesh TriFaceId; Delete sceneData. mMesh. pMesh TextureId; Delete sceneData. mMesh. pTextureUV; Delete sceneData. mMesh. mImage. data; Delete sceneData.mIncreaseMesh.pVertex; Delete sceneData.mIncreaseMesh.pMeshTriFaceId; Delete sceneData. mIncrementMesh. pWeightSet; //**************************** } RegisterSceneDataCallback (SDKWrapperSceneDataCallBack, this)// Registering Callbacks

# Video data analysis

SDKCWRAPPER_ API int WINAPI Sn3DUninstallGetImagasCallBack（ Sn3DGetImagesCallBack imageCallback, Void * owner ）;

Parameters

ImageCallback

[in] Video callback function

Owner

[in] class this pointer

Return Values

Reference error code definition

Remarks

1. Register the video callback function imageCallback;

2. It is recommended to register a callback after scanning is enabled;

SDKCWRAPPER_ API int WINAPI Sn3DStartProcessVideoData（ ）;

Return Values

Reference error code definition

Remarks

1. Start the video stream and successfully return 0;

SDKCWRAPPER_ API int WINAPI Sn3DTopProcessVideoData（ ）;

Return Values

Reference error code definition

Remarks

1. The video stream ends and returns 0 successfully;

Example: #Include<opencv2/opencv. hpp> Void Callback SDKWrapperVideoCallback (int camId, const unsigned char * data, int width, int height, int channel, void * pUserData) { //Analyze the current frame of the video stream, where data is the current frame data pointer, width is the image width, height is the image height, and channel is the image channel Cv:: Mat image; If (channel==3) cv:: Mat img (height, width, CV_8UC3, data); Else if (channel==1) cv:: Mat img (height, width, CV_8UC1, data); If (camId==0) cv:: imshow ("leftImage", img)// Camid=0 is the left image Else if (camId==1) cv:: imshow ("RightImage", img)// Camid=1 is the right image Cv:: waitKey (0); Return; } InstallGetImagasCallBack (SDKWrapperVideoCallback, this)// Register video stream callback Sn3DStartProcessVideoData()// Enable video streaming //... Sn3DTopProcessVideoData()// End video stream

# matters needing attention

There are many devices supported by the 3D SDK 2.0, and there are also some differences in the interfaces of different devices. If there are any inconsistencies in the documentation and demo code, please refer to the demo code for accuracy.

SDK2.0 only supports English for operations involving file names due to functional requirements. Subsequent versions will include support for different languages.

If you have any questions, please contact our company: support@shining3d.com .